Requirements Document

Project Title:

Speed Limit Sign Detection and Driver Alert App

Purpose:

To provide a smartphone or embedded application that enhances driver awareness by detecting UK road speed limit signs in real time and alerting the driver if the detected sign differs from the official speed for that road segment.

### 1. Functional Requirements

| ID | Requirement | Description |
| --- | --- | --- |
| FR-01 | Speed Sign Detection | The app must use a camera to identify and read UK circular speed limit signs in real time, with a minimum reliable detection range of ≥30 metres at low speeds (urban) and ≥100 metres at highway speeds. The optimal operational range should fall between 50–150 metres to ensure a pre-warning time of at least 1.5–3 seconds at normal driving speeds.. |
| FR-02 | Display Detected Speed | The detected sign value (e.g. 20, 30, 40, 50, 70 mph) must be displayed clearly on-screen. |
| FR-03 | Map Speed Retrieval | The app must obtain the official road speed limit from Google Maps or equivalent data source for the current GPS position. |
| FR-04 | Automatic Comparison | The detected sign value must automatically be compared with the map speed limit. |
| FR-05 | Driver Alert System | When a discrepancy or overspeed condition is detected, the app must issue an alert (sound, vibration, or on-screen warning). |
| FR-06 | Offline Functionality | The app must still detect signs and measure GPS speed when offline; map comparison should update once reconnected. |
| FR-07 | Temporary Sign Recognition | Temporary signs (e.g. roadworks) must take priority over map data when both are visible. |
| FR-08 | Data Privacy | The app must not record, store, or transmit video or GPS data externally; all processing should occur on the device. |
| FR-09 | Safe Operation Mode | The app must be designed to run hands-free once started before driving. The driver should start the app before setting off, after which it will operate autonomously without user interaction while driving.. |
| FR-10 | User Interface | The display must show detected limit, map limit, current speed, and visual warning status. |

### 2. Non-Functional Requirements

| Category | Requirement |
| --- | --- |
| Performance | The system must detect and classify a visible speed sign within 1 second of it appearing in view. Frame rate must remain ≥30 FPS under normal operation and at least 24 FPS in the worst-case scenario with OCR active.. |
| Accuracy | Detection accuracy must achieve ≥95% correct value recognition for standard daylight conditions and ≥90% under night or adverse weather. False positive rate for non-speed signs must remain ≤1%. |
| Usability | The interface must be simple, with minimal interaction required during operation (e.g., a single “Start Detection” button). |
| Reliability | The app must operate continuously for 24 hours or longer without freezing, crashing, or notable performance degradation.. |
| Compatibility | The app must function on standard modern smartphones or embedded devices with a camera and GPS (e.g., Android, iOS, Raspberry Pi). |
| Safety | The app must not distract the driver and must run autonomously after being started, without the need for physical interaction during operation.. |
| Privacy | All processing must be performed locally; no video or personal data should be transmitted or stored. |
| Maintainability | The detection model or datasets should be updatable without requiring a full reinstall. |
| Scalability | The system design must allow future integration with additional sensors or APIs. |

3. Engineering and Algorithmic Standards

Minimum Acceptable Detection Range (for reliable operation):

- ≥ 30 metres at low speed (urban driving).

- ≥ 100 metres at highway speeds.

Optimal Detection Range:
Between 50–150 metres, ensuring at least 1.5–3 seconds of driver pre-warning time at normal driving speeds.

4. Typical Detection Distance Requirements

| Context | Speed Limit | Required Detection Distance | Rationale |
| --- | --- | --- | --- |
| Urban roads | 20–40 mph (30–60 km/h) | 30–60 metres | Provides 2–3 seconds of reaction time for both driver and app to process. |
| Suburban/Main roads | 50–60 mph (80–100 km/h) | 80–120 metres | Allows time to confirm and adjust before reaching the sign. |
| Motorways | 70 mph (110–120 km/h) | 150–250 metres | Long-range detection ensures both system and driver can respond safely. |

Detection reliability must remain ≥90% within these ranges under daylight and clear visibility conditions.

5. Expected Outcomes

- A fully functional prototype capable of detecting UK speed limit signs between 30–250 metres depending on road context.

- Detection accuracy ≥95% in daylight, ≥90% in poor conditions, with a false positive rate ≤1%.

- Sustained ≥30 FPS real-time inference speed.

- Hands-free operation after activation, designed for continuous 24-hour reliability.

- Safe, privacy-compliant operation suitable for controlled or real-world evaluation.